# Landsat 8-9 OLI/TIRS Collection 2 Level-2

Landsat 8-9 OLI/TIRS Collection 2 Level-2

### 1. What is it?

- Name: Landsat 8-9 OLI/TIRS Collection 2 Level-2 Science Product.

- Source: USGS Earth Explorer.

- Type: A multi-spectral satellite image that captures both Thermal Infrared (Heat) and Optical (Visual light) data.

- Resolution: 30 meters per pixel (approx. the size of a basketball court).

- Timing: Captures Dubai at ~10:30 AM local time.

### 2. Why did we choose this specifically?

- "Surface" vs. "Top of Atmosphere": We specifically chose Level-2 data because it is already Atmospherically Corrected. USGS scientists have removed the interference from humidity and ozone, so the temperature readings represent the actual ground (road/roof), not the air above it.

- Resolution: 30m is the "Goldilocks" size—sharp enough to distinguish a highway from a park, but broad enough to cover the entire city in one file.

- Recency: As verified, we have access to Summer 2025 data, ensuring the project is current.

### 3. How do we read the data? (Structure)

The downloaded folder contains separate .TIF files (grayscale images) for each "Band" of light. It is not one color photo.

- _ST_B10.TIF: Surface Temperature Band. This is the raw heat data (currently looks like a grey square).

- _SR_B4 (Red) & _SR_B5 (NIR): Reflectance Bands. Used to see vegetation.

- _SR_B2, B3 (Blue, Green): Visible Bands. Used to see surface brightness/color.

- _MTL.txt/json: Metadata. The "Decoder Ring" containing the math coefficients needed to convert the raw numbers into Celsius.

- _QA_PIXEL: Quality Mask. Used to filter out clouds so they don't look like snow.

### 4. How exactly are we going to use it?

This dataset is the Foundation of your Machine Learning model.

- Target ($Y$): We will convert _ST_B10 into Degrees Celsius. This becomes the "Truth" our AI tries to learn.

- Features ($X$): We will perform math on the Optical bands to create new layers:

  - NDVI ($X_4$): (B5 - B4) / (B5 + B4) $\rightarrow$ Tells the AI where the plants are.

  - Albedo ($X_3$): Weighted sum of B2, B3, B4, B5 $\rightarrow$ Tells the AI which roofs/roads are Dark (Hot) vs. Bright (Cool).

- Why it is useful: It provides the high-resolution Structure (Where things are) that allows us to "sharpen" the blurry afternoon data later.

—-------

How to access the data:

### Step 1: Define Your Search Area

- Go to earthexplorer.usgs.gov.

- On the left panel under the "Search Criteria" tab:

  - Geocoder: Select "Path/Row".

  - Path: Enter 160.

  - Row: Enter 043.

  - Click "Show". You should see a pin drop exactly on Dubai.

- Date Range:

  - Enter a recent summer range to capture peak heat (e.g., 06/01/2024 to 09/30/2024).

- Cloud Cover:

  - Click the "Cloud Cover" tab (sub-tab under Search Criteria).

  - Set the slider to "Less than 10%". (We want clear ground pixels).

### Step 2: Select the Correct Dataset

This is where most students make a mistake. Do NOT select Level 1.

- Click the "Data Sets" button at the bottom.

- Expand the tree menu exactly like this:

  - > Landsat

  - > Landsat Collection 2 Level-2

  - ☑ Landsat 8-9 OLI/TIRS C2 L2

- Click "Results".

### Step 3: Verify & Download

- You will see a list of images. Look at the thumbnail image on the left.

  - Check: Is Dubai visible? Is it free of clouds over the city? (Clouds over the ocean are fine).

- Find the Green Download Icon (looks like a hard drive with a green arrow).

- Click it. A menu will pop up with "Product Options."

- Select "Product Bundle" (usually the largest file, ~800MB - 1GB).

  - Note: This downloads a .tar or .tar.gz compressed file.

### Step 4: Unpack the Data

- Unzip the downloaded file.

- Verify Contents: You are looking for these specific files to confirm you have the right data for your code:

  - ..._ST_B10.TIF (This is your Surface Temperature).

  - ..._SR_B4.TIF (Red Band).

  - ..._SR_B5.TIF (Near Infrared Band)

# VIIRS 750m

VIIRS/NPP Land Surface Temperature and Emissivity 6-Min L2 Swath 750m V002

#### 1. What is it?

- Name: VIIRS/NPP Land Surface Temperature and Emissivity 6-Min L2 Swath 750m V002 (Product ID: VNP21).

- Source: NASA Earthdata Search (LP DAAC).

- Satellite: Suomi NPP (National Polar-orbiting Partnership).

- Type: A "Swath" product containing thermal infrared measurements calibrated to Land Surface Temperature.

- Resolution: 750 meters per pixel (Mid-Resolution).

  - Context: One pixel covers a large neighborhood (e.g., all of Downtown Dubai). It is 25x coarser than Landsat but captures a much wider area.

- Timing: Captures Dubai twice daily:

  - Day: ~1:30 PM (Peak Heat).

  - Night: ~1:30 AM (Maximum Urban Heat Island Intensity).

#### 2. Why did we choose this specifically?

- The "Peak Heat" Gap: Landsat passes at 10:30 AM, which misses the hottest part of the day. VIIRS passes at 1:30 PM, giving us the critical "Maximum Temperature" data needed to simulate heat stress.

- The "Night" Gap: Urban Heat Islands are most visible at night when concrete stays hot while sand cools. VIIRS provides the 1:30 AM data essential for this analysis.

- Reliability vs. ECOSTRESS: While ECOSTRESS has higher resolution (70m), its orbit is irregular and swath is narrow (often missing parts of Dubai). VIIRS VNP21 has a 3,000 km swath, guaranteeing daily, full-map coverage of the entire UAE without "missing corners" or gaps.

- Standard Science Product: We chose the Standard (V002) product over the "Near Real Time" (NRT) version because NRT data is deleted after 7 days, whereas this product is permanently archived for scientific reproducibility.

#### 3. How do we read the data? (Structure)

- File Format: HDF5 (.h5 or .he5). This is a hierarchical container, not a simple image file. You cannot double-click to open it; you must use Python libraries like h5py or xarray.

- Key Data Arrays (Variables):

  - LST (Land Surface Temperature): The main heat layer.

    - Note: It is stored as a packed integer. You must multiply by the Scale Factor (0.02) to get Kelvin.

  - QC (Quality Control): A bitmask used to filter out clouds, poor-quality pixels, and dust storm interference.

  - latitude & longitude: Unlike Landsat (which is a grid), this is a "Swath." These arrays tell you the exact GPS coordinate of every pixel, which is required for georeferencing (un-rotating) the image.

  - Emis_14, Emis_15, Emis_16: Emissivity values for the thermal bands, derived from the split-window algorithm.

#### 4. How exactly are we going to use it?

This dataset is the Intensity Engine for your machine learning model.

- Training Target ($Y_{peak}$ & $Y_{night}$): We will use the 1:30 PM and 1:30 AM temperature readings as the "Truth" for how hot specific areas get.

- The Downscaling Process:

  - Blur: We degrade our high-res Landsat map (30m) to match the VIIRS resolution (750m).

  - Train: The AI learns the relationship: "At 1:30 PM (VIIRS), a 750m block with 80% Asphalt (Landsat) reaches 55°C."

  - Sharpen: We apply this rule back to the sharp 30m Landsat map to generate a Synthetic 30m Peak Heat Map.

- Why it is useful: It allows us to have the sharpness of a 30m map with the timing of a 1:30 PM map, effectively creating a satellite image that doesn't physically exist.

### How to access the data:

#### Step 1: Define Your Search

- Go to search.earthdata.nasa.gov and log in.

- In the top-left search bar, type: VNP21.

  - Important: Do NOT type "NRT". We want the standard science product.

- Select the collection named: "VIIRS/NPP Land Surface Temperature and Emissivity 6-Min L2 Swath 750m V002".

#### Step 2: Apply Filters (Location & Time)

- Spatial: Use the Rectangle Tool on the map to draw a box around Dubai.

- Temporal: Click the "Temporal" button.

  - Set Start/End Dates to a specific summer period (e.g., August 15, 2025 to August 16, 2025).

  - Note: Do not select "Today." Standard data takes ~3 days to process, so select a date at least one week in the past.

#### Step 3: Select the "Golden" Files

You will see a list of files on the left. You need two specific files based on their timestamps (UTC). Dubai is UTC+4.

- Find the Day File (Peak Heat):

  - Look for a time around 09:00 – 10:00 UTC (1:00 PM – 2:00 PM Dubai).

  - Example: VNP21.A2025227.0924...

- Find the Night File (UHI):

  - Look for a time around 21:00 – 22:00 UTC (1:00 AM – 2:00 AM Dubai).

  - Example: VNP21.A2025227.2148...

#### Step 4: Download

- Click the Green (+) button next to your chosen Day and Night files.

- Click the green "Download All" button at the bottom.

- Select "Direct Download" and save the .h5 files to your project folder.

# Sentinel-2 MSI

Sentinel-2 MSI (Multi-Spectral Instrument) Level-2A

#### 1. What is it?

- Name: Sentinel-2 MSI Level-2A (Bottom of Atmosphere Reflectance).

- Source: USGS Earth Explorer (Mirrored from ESA).

- Type: A high-resolution optical satellite image that captures visible and near-infrared light (like a super-camera), but does not measure temperature.

- Resolution: 10 meters per pixel (approx. the size of a large bus). This is 3x sharper than Landsat.

- Timing: Captures Dubai approximately every 5 days at ~10:30 AM local time.

#### 2. Why did we choose this specifically?

- The "Structure" Provider: While Landsat gives us the Temperature, 30m pixels are too blocky to distinguish a narrow sidewalk from a road. Sentinel-2’s 10m resolution provides the sharp "lines" we need to "sharpen" (downscale) the thermal maps.

- Atmospheric Correction: We specifically choose Level-2A because ESA (European Space Agency) has already removed the atmospheric haze, giving us pure "Surface Reflectance" values essential for accurate Albedo calculations.

- Vegetation Precision: The 10m resolution allows us to detect small patches of greenery (like median strips or home gardens) that Landsat (30m) would miss entirely.

#### 3. How do we read the data? (Structure)

The folder structure is more complex than Landsat. It uses the .SAFE format, which is a folder masquerading as a file.

- File Format: JPEG 2000 (.jp2). This compresses data efficiently but requires specific libraries (like rasterio or opencv) to read.

- Key Bands (Located in GRANULE/ -> IMG_DATA/ -> R10m/):

  - _B02.jp2 (Blue): Used for Albedo.

  - _B03.jp2 (Green): Used for Albedo.

  - _B04.jp2 (Red): Used for NDVI and Albedo.

  - _B08.jp2 (NIR - Near Infrared): The critical band for vegetation. (Note: It is Band 8 here, unlike Band 5 in Landsat).

#### 4. How exactly are we going to use it?

This dataset provides the High-Resolution Features ($X$) for your Machine Learning model.

- Features ($X$): We will perform math on these 10m bands to create "Feature Maps":

  - NDVI ($X_4$): (Band 8 - Band 4) / (Band 8 + Band 4) $\rightarrow$ Tells the AI exactly where the plants are with high precision.

  - Albedo ($X_3$): Weighted sum of B2, B3, B4, B8 $\rightarrow$ Tells the AI which surfaces are "Bright/Reflective" (Cool Roofs) vs. "Dark/Absorbent" (Asphalt).

- Why it is useful: By feeding this 10m data into the model, we force the AI to predict temperature at 10m (or 30m sharpened) resolution, creating a map that looks much more detailed than the raw thermal data allows.

### How to access the data:

### Step 1: Go to the Copernicus Browser

- Navigate to: browser.dataspace.copernicus.eu

- Login: You will need to create a free account (top right corner).

### Step 2: Set Up Your Search

- Search Bar (Top Left): Type "Dubai" and press Enter. Select "Dubai, United Arab Emirates" to zoom the map.

- Time Range (Top Center): Click the calendar.

  - Select August 2025 (e.g., 2025-08-01 to 2025-08-31).

- Data Source (Left Menu):

  - Uncheck everything except Sentinel-2.

  - Under Sentinel-2, check Level-2A (this is the atmospherically corrected "Surface Reflectance" you need).

  - Set Cloud Cover: Slide the bar to < 10%.

### Step 3: Visual Inspection & Selection

- You will see yellow tiles appear on the map.

- Click on a tile that covers Dubai City (look for Tile ID: 40RCN).

- Check the Image: The browser shows you a high-res preview instantly.

  - Does it cover the Palm and Sheikh Zayed Road?

  - Is it cloud-free?

  - Note: Sometimes the tile edge cuts through the city. If so, check the neighboring tile (40RBN). You might need to download two adjacent tiles and merge them (Python merge function) if your study area crosses the line.

### Step 4: Download

- Once you find the perfect tile, click the "Download" icon (looks like an arrow pointing down) in the right-hand panel.

- Product: Ensure you are downloading the Product (Complete zip file), not just the "Image" (jpeg). It should be around 800MB - 1GB.

# OSM

OpenStreetMap (OSM) Data via osmnx

#### 1. What is it?

- Name: OpenStreetMap (OSM) Data - Dubai Region.

- Source: Accessed programmatically via the osmnx Python library (which queries the Overpass API).

- Type: Vector Data (Lines for roads, Polygons for buildings, Points for amenities). unlike the "Raster" (pixel) data from satellites.

- Resolution: Infinite/Vector Precision. Unlike a satellite image which is limited to 10m or 30m pixels, vector lines represent the exact mathematical shape of the physical infrastructure.

- Timing: Live, community-updated database (snapshot taken at the moment you run the code).

#### 2. Why did we choose this specifically?

- The "Ground Truth" Labeler: Satellites can tell us a surface is "Gray," but they struggle to know if that gray patch is a Road or a Warehouse Roof. OSM explicitly labels objects (highway=primary, building=apartments), removing the ambiguity.

- Graph Capability: Unlike a static shapefile or image, OSM data downloaded via osmnx comes structured as a Network Graph (Nodes & Edges). This is mandatory for your "Phase 4: Citizen Routing" engine because you need a connected mathematical graph to run Dijkstra's pathfinding algorithm.

- Completeness in Dubai: Dubai has a highly active OSM community. The road network is near-perfect, and building footprints in the urban core are highly detailed, making it a reliable source for density calculations.

#### 3. How do we read the data? (Structure)

You do not download a file manually (like a zip). You write Python code to stream it directly into your script. The data arrives in two specific Python structures:

- Roads (MultiDiGraph): A directional graph object.

  - Nodes: Intersections (Lat/Lon points).

  - Edges: Streets (containing metadata like length, lanes, maxspeed).

- Buildings (GeoDataFrame): A tabular format (like an Excel sheet with shapes).

  - Geometry: Polygons defining the exact outline of every building.

  - Tags: Metadata like height, building:levels, or type.

#### 4. How exactly are we going to use it?

This dataset provides the Physical Features ($X_1, X_2$) for your Machine Learning model and the Navigation Network for the app.

- Features ($X$): We calculate "Density" metrics for every hexagon:

  - Road Density ($X_1$): Sum(Length of Road Segments * Estimated Width) $\rightarrow$ Tells the AI exactly how much asphalt is in that grid cell.

  - Building Density ($X_2$): Sum(Area of Building Footprints) $\rightarrow$ Tells the AI how much concrete mass exists.

- Masking: We use the road network to create an "Asphalt Mask," ensuring we only compare road temperatures to road pixels.

- Routing: The raw Graph object (G) becomes the foundation for the "Cool Path" algorithm in Phase 4.

### How to access the data:

#### Step 1: Install the Library

You do not visit a website. You use your Python environment.

- Command: pip install osmnx geopandas matplotlib

#### Step 2: The "Data Harvester" Script

You run a Python script to query the database for Dubai's specific bounding box.

Code Logic:

- Define Area: Set the query location to "Dubai, United Arab Emirates".

- Fetch Roads: Use ox.graph_from_place(place_name, network_type='drive') to get the asphalt network for your Heat Map.

- Fetch Walkways: Use ox.graph_from_place(place_name, network_type='walk') to get the pedestrian network for your Citizen App.

- Fetch Buildings: Use ox.features_from_place(place_name, tags={'building': True}) to get the footprints.

#### Step 3: Save for Later

Since downloading takes 2-3 minutes every time, you save the data locally immediately after the first download.

- Save Graph: ox.save_graphml(G, "dubai_network.graphml")

- Save Buildings: gdf.to_file("dubai_buildings.gpkg", driver="GPKG")

Verification:

- Visual Check: Plot the graph using ox.plot_graph(G).

- Check: Do you see the Palm Jumeirah's fronds? Do you see the grid of Sheikh Zayed Road? If yes, the data is valid.

# Overture Maps

Overture Maps

#### 1. What is it?

- Name: Overture Maps "Buildings" Theme (Release 2025-11-19.0).

- Source: Amazon S3 / Azure Open Datasets (Accessed via Python duckdb).

- Type: Vector Data (Polygons with detailed 3D attributes).

- Resolution: Infinite/Vector Precision. Similar to OSM, but optimized for cloud-native big data processing.

- Timing: Monthly releases (Current version: November 2025).

#### 2. Why did we choose this specifically?

- The "Missing Height" Solution: While OpenStreetMap is excellent for 2D footprints, it often lacks height data for non-landmark buildings in Dubai (like residential villas or warehouses). Overture fills these gaps by aggregating data from Microsoft (AI-detected heights) and Esri Community Maps.

- Advanced Data Engineering: Accessing data via GeoParquet files and DuckDB (SQL on Cloud Storage) demonstrates advanced "Big Data" engineering skills, adding significant technical weight to your Final Year Project compared to simply downloading a standard shapefile.

- Completeness: It combines multiple open data sources into a single schema, providing a more robust building inventory than any single source alone.

#### 3. How do we read the data? (Structure)

You do not download a file from a website. You query the cloud database directly using Python.

- Format: GeoParquet (A highly efficient, compressed column-store format optimized for cloud queries).

- Query Engine: We use duckdb, a fast in-process SQL database that can read data directly from Amazon S3 without downloading the entire global dataset.

- Key Attributes:

  - geometry: The binary (WKB) polygon shape of the building.

  - height: Explicit height in meters (Critical for shadows).

  - num_floors: Floor count (Used as a fallback to calculate height: Floors × 3.5m).

#### 4. How exactly are we going to use it?

This dataset acts as the 3D Geometry Layer for both your Macro and Micro models.

- Feature $X_5$ (Urban Geometry): We calculate the Average Building Height for every hexagon in the grid. This allows the Machine Learning model to understand the "Urban Canyon Effect" (distinguishing between hot, flat parking lots and cool, shaded skyscraper districts).

- Shadow Analysis (Citizen View): In Phase 4, we use these exact height values to calculate Shadow Lengths using trigonometry, which determines the "Thermal Comfort" score for walking routes.

- 3D Visualization: It enables the extrusion of buildings in your final dashboard for a professional "Digital Twin" look.

### How to access the data:

#### Step 1: Install the Libraries

You do not visit a website. You use your Python environment to connect to the cloud.

- Command: pip install duckdb geopandas shapely matplotlib

#### Step 2: The "Height Harvester" Script

You run a Python script to execute a SQL query against the Overture S3 bucket, filtering strictly for Dubai's coordinates.

Code Logic:

- Define Bounding Box: Set coordinates for Dubai (Min X: 54.9, Min Y: 24.8, Max X: 55.6, Max Y: 25.3).

- Connect: Initialize a duckdb connection with the httpfs (HTTP File System) and spatial extensions.

- Execute Query:
SELECT height, num_floors, geometry FROM read_parquet('s3://overturemaps.../theme=buildings...') WHERE bbox intersects Dubai.

- Safe Parsing: Use a custom safe_wkb_load function to handle any corrupted binary geometries from the cloud stream.

#### Step 3: Process & Save

Since the query returns raw data, you convert it into a usable format and save it immediately.

- Convert: Transform the binary geometry into Shapely objects using geopandas.

- Filter: Drop empty or corrupted rows.

- Save: gdf.to_file("dubai_overture_buildings.gpkg", driver="GPKG")

Verification:

- Data Quality Report: The script prints the percentage of buildings with valid 3D data (Target: >20-30%).

- Visual Check: Plot the data using gdf.plot. You should see "Hot Spots" (Red points) in Downtown Dubai and Dubai Marina representing tall towers, confirming the 3D data is valid.

# GHS-OBAT

#### GHS-OBAT

#### 1. What is it?

- Name: GHS-OBAT R2024A (Global Human Settlement - Open Buildings Attribute Table).

- Source: European Commission – Joint Research Centre (JRC) / Copernicus.

- Type: Tabular Data (CSV containing Attributes for Building Footprints).

- Content: A global database that assigns "missing" attributes (Height, Age, Function) to 2.3 billion building footprints by fusing satellite imagery (Sentinel-2) with the Overture Maps geometry.

- Key Attributes: height (Average Gross Building Height in meters), use (Residential vs. Non-Residential), and epoch (Construction year).

#### 2. Why did we choose this specifically?

- The "Fallback" Layer: While Overture Maps provides perfect 2D shapes, it often lacks 3D height data for standard buildings (like villas or warehouses). GHS-OBAT fills this specific gap by using satellite-derived statistical estimates for every building.

- Scientific Consistency: It is produced by the European Commission's JRC, providing a scientifically validated "best guess" for building heights where official data or direct AI detection fails.

- Global Coverage: Unlike local municipal datasets which might be restricted, this dataset covers 100% of Dubai, ensuring that even remote industrial areas (like Jebel Ali) have valid height data for your shadow model.

#### 3. How do we read the data? (Structure)

You download a specific tile (CSV file) relevant to your region. You read it using Python (pandas or duckdb).

- Format: CSV (Comma Separated Values).

- Query Engine: We use duckdb or pandas to read the file and perform a Spatial Join with our Overture polygons.

- Key Attributes:

  - height: The estimated height in meters (Critical for Priority 2 in our Fusion Logic).

  - lon / lat: The coordinate centroid of the building (used to link this CSV row to the Overture Polygon).

#### 4. How exactly are we going to use it?

This dataset acts as the Secondary Logic Layer (Priority 2) for your 3D City Model.

- Data Fusion (Priority 2): In your "Fusion Script," if Overture Maps does not have a height for a building, the code checks this GHSL dataset.

  - Logic: COALESCE(Overture_Height, GHSL_Height)

- Urban Morphology ($X_5$): It helps fill in the "Urban Geometry" feature for the Machine Learning model, ensuring that no part of the Heat Map has "Zero Height" (which would break the shadow calculations).

### How to access the data:

#### Step 1: Download the Tile

- Source: JRC GHSL Data Repository. (https://data.jrc.ec.europa.eu/dataset/f41a22f1-5741-4c41-86eb-6384654f6927)

- Selection: Download the tile corresponding to the UAE/Middle East (GHS_OBAT_CSV_ARE...).

- File: You should have a .csv file (e.g., GHS_OBAT_CSV_ARE_E2020_R2024A_V1_0.csv).

#### Step 2: The "Spatial Join" Script

You run a Python script to link these CSV points to your Overture Polygons based on location.

- Code Logic:

  - Load: Read the CSV into a DuckDB table or Pandas DataFrame.

  - Filter: Select only rows within the Dubai Bounding Box (Min X: 54.9, Max X: 55.6).

  - Join: Perform a Spatial Join where the GHSL Point falls inside the Overture Polygon.

  - Query: SELECT * FROM overture_buildings LEFT JOIN ghsl_points ON ST_Intersects(...)

#### Step 3: Process & Save

- Merge: The script updates the height column in your main GeoDataFrame.

  - Rule: Only update if the original Overture height was NULL.

- Save: The result is saved into your master dubai_fused_inventory.gpkg file.

#### Verification:

- Data Quality Report: The script prints the "Enrichment Rate" (e.g., "GHSL provided heights for 45,000 buildings that Overture missed").

- Visual Check: Plot the map coloring buildings by source. You should see Overture (Red) for major towers and GHSL (Blue) for the surrounding villas/warehouses.

# DM Building Summary (via Makani Integration)

Dubai Municipality Building Summary (via Makani Integration)

#### 1. What is it?

- Name: dm_building_summary_information-open (Enriched via Makani API).

- Source: Dubai Pulse (Open Data Portal) & Makani Smart Addressing System (API).

- Type: Tabular Data (CSV) augmented with Geospatial Point Data (JSON).

- Content: The official government registry of buildings in Dubai, containing verified structural details such as floor counts and usage types. Since the raw CSV lacks coordinates, it is "enriched" using a custom script that queries the Makani system to retrieve the precise location (Lat/Lon) for each building.

- Key Attributes: typical_floors_count (Official vertical data), parcel_id (The linking key), and makani_number (Geospatial identifier).

#### 2. Why did we choose this specifically?

- The "Ground Truth" Layer: Unlike Overture (AI estimates) or GHSL (Satellite probability), this dataset contains the actual permitted floor counts filed with the municipality. It serves as the "Gold Standard" for accuracy.

- Correction Mechanism: It allows us to correct the "AI Hallucinations" in other datasets. If Overture thinks a building is 50m tall but the DM registry says it is a G+1 villa, the DM data overrides the error, preventing false shadow calculations.

- Official Integration: Using government-sourced data aligns the project directly with the RTA/Municipality's own operational standards, increasing the project's validity for stakeholders.

#### 3. How do we read the data? (Structure)

We process this in two stages: first reading the static CSV, then dynamically fetching coordinates.

- Format: CSV (Base Data) + JSON (Location Data from API).

- Query Engine: Python pandas for the CSV and requests for the Makani API.

- Key Attributes:

  - typical_floors_count: The core variable used to calculate precise height (Floors * 3.5m).

  - parcel_id: The unique identifier used to query the Makani API for location.

  - LAT / LNG: The coordinate output from the Makani "Bridge" script, enabling spatial joining.

#### 4. How exactly are we going to use it? (We’ll use it prolly only for a few hotspots with high number of tall buildings due to the api rate limits)

This dataset acts as the Priority 1 Logic Layer for your 3D City Model.

- Data Fusion (Priority 1): In your "Fusion Script," this is the first source checked.

  - Logic: IF DM_Floors EXISTS -> Height = DM_Floors * 3.5m (Overrides everything else).

- Validation Set: It serves as a benchmark to test the accuracy of the GHSL and Overture datasets. (e.g., "We validated our AI model against 5,000 official government records").

- Shadow Precision: By providing exact floor counts, it ensures that the shadow analysis for critical pedestrian corridors (like those near Metro stations) is based on real physical constraints, not estimates.

### How to access the data:

#### Step 1: Download the Base Data

- Source: Dubai Pulse Open Data Portal.

- Selection: Download dm_building_summary_information-open.

- File: You will have a large .csv file containing building metadata but no map coordinates.

#### Step 2: The "Makani Bridge" Script

You run a Python script to fetch the missing coordinates for your target buildings.

(https://www.makani.ae/desktop)

- Code Logic:

  - Load: Read the CSV into Pandas.

  - Filter: Select high-priority buildings (e.g., commercial towers or buildings in your study area).

  - Query: Loop through parcel_ids and send a request to the Makani API (https://www.makani.ae/.../SmartSearchResult).

  - Extract: Parse the JSON response to get the SHAPE (Latitude/Longitude).

#### Step 3: Process & Save

- Spatial Join: Convert the fetched coordinates into a GeoDataFrame (Points) and join them with Overture Polygons (Shapes).

- Enrich: Inject the typical_floors_count into the Overture dataset.

- Save: The verified records are saved as the "High Confidence" layer in your dubai_fused_inventory.gpkg.

#### Verification:

- Data Quality Report: The script reports the "Match Rate" (e.g., "Successfully geolocated 3,500 priority buildings via Makani").

- Visual Check: Plot the points on top of the satellite map. They should align perfectly with building centers, confirming the bridge script worked.

# dm_communities-open

Dm_communities-open

#### 1. What is it?

- Name: dm_communities-open (Dubai Communities).

- Source: Dubai Pulse (Official Open Data Portal of Dubai).

- Type: Vector Polygons (Shapefile/KML).

- Content: A geospatial file defining the official legal boundaries of every community (district) in Dubai (e.g., "Al Barsha 1", "Jebel Ali Industrial", "Al Marmoom"). It serves as the administrative skeleton of the city.

- Key Attributes:

  - COMMUNITY_ID (Unique Code, e.g., "373").

  - CNAME_E (English Name, e.g., "AL BARSHA FIRST").

  - SHAPE (Polygon Geometry).

#### 2. Why did we choose this specifically?

- The "Desert Paradox" Solution: As discussed, satellite thermal sensors see open desert sand as "Hot" during the day. If you do not mask these areas out, your AI model will incorrectly recommend "Cool Pavement" interventions for empty sand dunes in the desert. This dataset allows us to spatially filter the analysis to the Urban Growth Boundary.

- Strategic Scoping (Policy Alignment): RTA and Municipality projects are budgeted and planned by district. An analysis that says "Pixel X is hot" is scientifically interesting but administratively useless. An analysis that says "Al Quoz Industrial 2 is the hottest district in Dubai" is an actionable policy insight.

- Sector-Based Logic: The ID system (e.g., 3xx for Coastal/Urban, 5xx for Jebel Ali) allows for programmatic classification of "Urban" vs "Rural" zones without manual drawing.

#### 3. How do we read the data? (Structure)

You download a standard GIS file that Python reads natively.

- Format: .kml (Google Earth) or .shp (Shapefile).

- Query Engine: geopandas (Python).

- Key Attributes:

  - CNAME_E: The label displayed on your dashboard (e.g., "Downtown Dubai").

  - geometry: The polygon used to clip/mask your satellite data.

  - COMMUNITY_ID: Used to filter sectors (e.g., "Keep IDs starting with 1-6, drop 7-9").

#### 4. How exactly are we going to use it?

This dataset acts as the Spatial Container for your entire analysis.

- Phase 1 (Masking): We use it to clip the satellite images.

  - Logic: satellite_data.clip(urban_communities_polygon)

- Phase 3 (Reporting): The dashboard will display a Choropleth Map where each district is colored by its "Heat Risk Score."

- User Interface: Users will select a district from a dropdown menu (populated by CNAME_E) to zoom into that specific area.

### How to access the data:

#### Step 1: Go to Dubai Pulse (https://www.dubaipulse.gov.ae/data/dm-location/dm_community-open)

- Navigate to: dubaipulse.gov.ae (Direct Link).

- Search: If the link changes, search for dm_community-open or look under "Location" -> "Dubai Municipality".

#### Step 2: Download

- Click on the dataset titled "dm_community-open".

- Look for the "KML" or "Shapefile" download button.

  - Note: The file is often named Community.kml or Community.zip.

  - Alternative: If the portal is down, osmnx.geocode_to_gdf("Dubai, UAE") fetches the emirate boundary, but the official DM file is required for the internal district divisions.

#### Step 3: The "District Filter" Script

Run this Python script to load the boundaries and filter out the desert.

Code Logic:

- Load: gdf = gpd.read_file("Community.kml")

- Filter Sectors: Dubai Sector IDs are the first digit of the Community ID.

  - Urban Sectors: 1xx (Deira), 2xx (Muhaisnah), 3xx (Bur Dubai/Jumeirah), 4xx (Al Warqa), 5xx (Jebel Ali), 6xx (Ras Al Khor).

  - Rural/Desert Sectors: 7xx, 8xx, 9xx.

- Action: urban_gdf = gdf[gdf['COMMUNITY_ID'].astype(str).str[0].isin(['1','2','3','4','5','6'])]

- Visualize: Plot the map to confirm it covers the city but excludes the deep desert.

# Distribution of Estimated Population

#### Distribution of Estimated Population

#### 1. What is it?

- Name: Population by Communities (Annual Bulletin).

- Source: Dubai Statistics Center (DSC) / Dubai Pulse.

- Type: Tabular Data (Excel/CSV).

- Content: The official demographic census providing the total resident population for each of Dubai’s planning communities (e.g., "Community 118 - Naif: 53,000 people").

- Key Attributes:

  - Community Number: The unique ID (e.g., 373) that acts as the "Foreign Key" to link with your Map.

  - Total Population: The number of active residents.

  - Gender Breakdown: (Optional) Useful for analyzing workforce-heavy industrial areas vs. family residential zones.

#### 2. Why did we choose this specifically?

- The "Human Cost" Factor: A heat map alone is just physics. Combining it with population turns it into a Risk Map. A 50°C temperature in the empty desert (Sector 9) matters less than a 45°C temperature in a densely populated district like Deira (Sector 1).

- ROI Justification: The RTA prioritizes investments where they benefit the most people. This data allows you to calculate "Cooling Benefit per Capita," proving that an intervention in a high-density area has a higher Return on Investment (ROI).

- Seamless Integration: Unlike global raster datasets (like GHS-POP) which are estimates, this is the official government census. It maps 1:1 with the dm_communities-open shapefile using the Community ID.

#### 3. How do we read the data? (Structure)

You download a spreadsheet (.xlsx or .csv). You read it using Python (pandas).

- Format: Standard Table.

- Linkage: You perform a Table Join (Merge) with the dm_communities shapefile.

  - Logic: Shapefile['COMMUNITY_ID'] == Census['Community Number'].

#### 4. How exactly are we going to use it?

This dataset drives the "Priority Scoring Algorithm" in your dashboard.

- Feature $X_{pop}$ (Population Density): We calculate Population / District_Area.

- Vulnerability Index:
$$Score = (Temperature \times 0.5) + (PopulationDensity \times 0.5)$$

- User Interface: When a user hovers over a district, the dashboard displays:

  - "District: Al Rigga"

  - "Avg Temp: 44°C"

  - "Population Affected: 86,000"

  - "Risk Level: CRITICAL"

### How to access the data:

#### Step 1: Go to Dubai Statistics Center (or Dubai Pulse) (https://www.dsc.gov.ae/en-us/Themes/Pages/Population-and-Vital-Statistics.aspx?Theme=42)

- Navigate to: dsc.gov.ae.

- Look for the "Population" theme.

- Select "Population by Community" (Annual Report).

  - Alternative: Search for "Population" on Dubai Pulse to get a machine-readable CSV directly.

#### Step 2: Download

- Download the Excel (.xlsx) or CSV file.

- Note: If only a PDF is available, you can easily copy-paste the table into Excel, as it is a clean list of ~200 communities.

#### Step 3: The "Population Merger" Script

Run this Python script to link the people to the map.

Code Logic:

- Load Map: gdf = gpd.read_file("dm_communities.shp")

- Load Data: df_pop = pd.read_excel("population_2023.xlsx")

- Clean IDs: Ensure both IDs are strings (e.g., "373" matches "373").

- Merge: merged_gdf = gdf.merge(df_pop, left_on='COMMUNITY_ID', right_on='Community_No')

- Calculate Density: merged_gdf['pop_density'] = merged_gdf['Total_Pop'] / merged_gdf.area

- Visualize: Plot a Choropleth Map based on Density to confirm that Deira/Bur Dubai light up as "High Density."

# RTA Metro Ridership & Infrastructure Data

#### RTA Metro Ridership & Infrastructure Data

#### 1. What is it?

- Name: Combined RTA Metro Data (Ridership + Infrastructure).

- Source: Dubai Pulse (Open Data Portal).

- Type: Hybrid Tabular (CSV) and Vector (KML/SHP) data.

- Content: A unified dataset merging the Demand (Passenger Volumes) with the Supply (Station Locations).

  - rta_metro_passengers_trips_by_station_monthly-open (CSV): Contains statistical ridership counts per station (e.g., "Union Station: 6.6 million riders/month").

  - rta_rail_platforms-open (KML/SHP): Contains the geospatial location (Lat/Lon) of each station platform.

- Key Attributes: METRO_STATION (Name), TRIPS (Passenger Volume), geometry (Point/Polygon).

#### 2. Why did we choose this specifically?

- The "Human Heat" Factor: A heat map tells you where it is hot, but not where it matters. A 50°C highway in the desert is less critical than a 45°C sidewalk outside a busy metro station. This data introduces "Pedestrian Vulnerability" into the model.

- Strategic Alignment: This directly supports the RTA's "First Mile/Last Mile" strategy. By prioritizing cooling interventions around high-traffic hubs (like Union or BurJuman), the project demonstrates operational relevance to RTA planners.

- Dynamic Prioritization: It allows the system to calculate a "Vulnerability Score." A moderately hot district with 20,000 daily pedestrians will be flagged as "Critical," while a hotter but empty industrial zone will be "Low Priority."

#### 3. How do we read the data? (Structure)

We process two files and join them in Python.

- Ridership File (CSV):

  - Columns: YEAR, MONTH, METRO_STATION, TRIPS.

  - Processing: We group by METRO_STATION to get the average monthly volume.

- Platforms File (KML):

  - Columns: Name, geometry (Point coordinates).

- The Join: We merge the two tables on the Station Name column (e.g., matching "Union" in CSV to "Union" in KML).

#### 4. How exactly are we going to use it?

This dataset drives the Priority Scoring Algorithm in your dashboard.

- Feature $X_{transit}$ (Transit Density): We create a "Pedestrian Pressure Score" for every hexagon.

  - Logic: If a hexagon is within 500m of a Metro Station, it inherits that station's Ridership Count.

- Weighted Heat Risk: The dashboard calculates a composite priority score using your formula:
$$Priority = (Temperature_{Normalised} \times 0.5) + (Population_{Normalised} \times 0.3) + (MetroUsers_{Normalised} \times 0.2)$$

- Impact: This ensures that your "Cool Corridor" recommendations are placed exactly where they benefit the maximum number of commuters.

### How to access the data:

#### Step 1: Download Ridership Data (https://www.dubaipulse.gov.ae/data/rta-rail/rta_rail_platforms-open, https://www.dubaipulse.gov.ae/data/rta-rail/rta_metro_passengers_trips_by_station_monthly-open)

- Go to: dubaipulse.gov.ae.

- Search: rta_metro_passengers_trips_by_station_monthly-open.

- Download: Select the CSV file. (Look for the most recent year, e.g., 2024/2025).

#### Step 2: Download Station Locations

- Search: rta_rail_platforms-open or rta_metro_stations-open.

- Download: Select the KML or Shapefile format.

#### Step 3: The "Transit Fusion" Script

You run a Python script to merge the volume data into the map.

Code Logic:

- Load: Read ridership.csv and stations.kml using pandas/geopandas.

- Clean: Standardize station names (e.g., fix "Burj Khalifa/Dubai Mall" spelling differences).

- Merge: gdf_stations = gdf_stations.merge(df_ridership, on='station_name').

- Buffer: Create a 500m buffer around each station point to define the "Walkshed."

- Spatial Join: Link these buffers to your Hexagon Grid.

- Normalize: Scale the ridership numbers (0.0 to 1.0) to fit into your Priority Formula.

# ECOSTRESS (NO USE)

### ECOSTRESS

#### 1. What is it?

- Name: ECOSTRESS Swath Land Surface Temperature and Emissivity Instantaneous L2 Global 70 m (Product ID: ECO_L2_LSTE).

- Source: NASA Earthdata Search.

- Satellite: ISS (International Space Station).

- Resolution: 70 meters (High-Resolution).

  - Context: This is a massive upgrade from VIIRS (375m). It is nearly as sharp as Landsat (30m), meaning you can see individual large buildings and highway interchanges directly in the thermal data.

- Timing: Variable (Precessing Orbit). It passes over Dubai at different times every day.

  - Your Files: You have successfully secured Peak Heat (~1:30 PM) and Late Night (~1:45 AM - 3:00 AM) files from Summer 2025.

#### 2. Why did we choose this specifically?

- The "Resolution Revolution": VIIRS (375m) is good, but ECOSTRESS (70m) is 5x sharper. This reduces the "blur" in your machine learning training data. Your model will learn the temperature of specific districts much more accurately because the pixels are smaller.

- The "Diurnal" Capability: Unlike Landsat (which is stuck at 10:30 AM), the ISS orbit allows ECOSTRESS to capture data at 1:30 PM (Max Heat) and 3:00 AM (Max UHI Intensity).

- Distinction-Level Quality: Using ISS data adds significant technical novelty to your project compared to standard "Landsat-only" student projects.

#### 3. How do we read the data? (Structure)

- Warning: Like VIIRS, this is Harder to open than Landsat.

- File Format: HDF5 (.h5).

- What to look for inside: You need Python libraries like h5py or h5netcdf.

- Key Data Arrays:

  - SDS/LST: The Scientific Data Set for Land Surface Temperature.

    - Note: Stored in Kelvin. Needs a scale factor of 0.02.

    - Formula: (DN * 0.02) - 273.15 = Celsius.

  - SDS/QC: Quality Control flags (Use this to hide clouds/errors).

  - SDS/Emis1 - Emis5: Spectral Emissivity (useful for validation).

  - Geolocation: Unlike GeoTIFFs, these files might store Latitude and Longitude as separate arrays (latitude, longitude) that you must "mesh" together in Python to project the map correctly.

#### 4. How exactly are we going to use it?

This dataset is the "Truth Anchor" for your Temporal Downscaling Model.

- The Training Goal: Create a synthetic 30m Peak Heat Map (sharpening the 70m data to 30m).

- The Process:

  - We input the 1:30 PM ECOSTRESS image (70m) as the "Target Heat Pattern."

  - We input the 10:30 AM Landsat Structure (Roads/Vegetation) as the "Feature Map."

  - We tell the AI: "Look at the relationship between Roads and Heat at 70m resolution."

- The Result: Because the input (70m) is so sharp, the AI learns a very tight correlation (e.g., "Sheikh Zayed Road is exactly 58°C at 1:30 PM"), allowing it to predict the temperature of smaller streets with high confidence.